OS-ASSIGNMENT

- Define cross-toolchain and explore what is inside a cross-compiling toolchain.

Sol:

Cross-Toolchain Definition:

- A set of tools for cross-compiling software.

- Enables building for a target architecture different from the host.

Components Inside a Cross-Compiling Toolchain:

- Cross-Compiler: Generates code for the target architecture.

- Linker: Connects and creates the final executable.

- Assembler: Translates assembly code to machine code.

- Standard C Library (libc): Adapted for the target system.

- Header Files: Target-specific declarations and definitions.

- Runtime Libraries: Support for executing programs.

- Loader: Loads and executes programs on the target.

- Debugging Tools: (e.g., GDB) for target-specific debugging.

- Read the datasheet, user manual/ getting started manual specific to your Embedded Linux-board. Write a 4 page summary based on your understanding about board details such as CPU architecture & type, memory types supported by the board, on-board buses, booting sequence, peripherals supported, various booting options and any other relevant information.

Sol:

CPU Architecture & Type:

- CPU: Texas Instruments Sitara AM3358/9 ARM Cortex-A8 processor.

- Architecture: ARMv7-A architecture.

- Memory Types Supported:

- RAM: 512MB DDR3.

- Storage: 4GB 8-bit eMMC on-board flash storage.

- MicroSD: External storage option.

- On-board Buses:

- GPIO: General Purpose Input/Output pins.

- I2C, SPI, UART: Various communication interfaces for connecting peripherals.

Booting Sequence:

- The BeagleBone Black can boot from different sources, including the on-board eMMC, microSD card, or USB.

Peripherals Supported:

- Ethernet: 10/100 Ethernet.

- USB: USB 2.0 ports for connecting peripherals.

- HDMI: HDMI interface for video output.

- Audio: Stereo output/input.

- CAN: Controller Area Network interface.

- PWM: Pulse Width Modulation for controlling motors and other devices.

Various Booting Options:

- eMMC: Booting from the on-board eMMC storage.

- microSD: Booting from a microSD card.

- USB: Booting from a USB device.

- Boot up an Embedded Linux Board, login into the system and grab a shell. Try basic Linux commands on the shell.

Sol:

- Download Debian Beaglebone Black Image File :-

https://www.beagleboard.org/distros/am335x-12-2-2023-10-07-4gb-microsd-iot

- Download Balena etcher for flashing the Image file SD card:

https://etcher.balena.io/#download-etcher

- Install the balena etcher and transfer the image to SD card

- Complete serial connection on the beagleboneboard

- Place the card inside the BBB; provide power supply to board

- $

- Use the shell commands to find details about CPU, Memory capacity, Memory and I/O maps, persistent storage details.

Sol:

1. CPU Details:

- $ cat  /proc/cpuinfo

  - Displays detailed information about the CPU, including model, architecture, and clock speed etc.

- $ lscpu

  - Provides detailed information about the CPU, including its architecture, number of cores, threads per core, and other relevant details in a concise format.

2. Memory Capacity:

- $ free -h

  - Shows information about system memory usage and capacity in a human-readable format.

3. Memory Map:

- $ cat /proc/iomem

  - Provides a list of current memory ranges used by different devices, showing the memory map of the system.

4. I/O Map:

  - $ lspci

    - Lists all PCI buses and devices connected to them, providing information about I/O devices and their addresses.

- Persistent Storage Details:

  - $ df -h

    - Displays information about disk space usage, including details about mounted filesystems and their capacities.

  - $ lsblk

  - Lists information about block devices, including disks and partitions, providing details about their sizes and mount points.